*******************************

* ICI PAINTS SAFETY DATASHEET *

*******************************

Date: 03/03/2003

1. IDENTIFICATION OF THE PREPARATION AND COMPANY

PRODUCT NUMBER: 017-3508

PRODUCT NAME  : Dulux Masa Szpachlowa Szybkoschnaca

Producent:                ICI Alabastine Hollandia B.V.

Hogesteeg 27e

5324 AA Ammerzoden, The Netherlands

Telephone       073-59999305

Fax             073-59999399

Emergency Telephone : 073-5999333

Dystrybutor  :          ICI Polska sp. z o.o. Oddział w Pilawie

ul.Przemysłowa 3 ;  08-440 Pilawa

2. COMPOSITION/INFORMATION ON INGREDIENTS

Certain ingredients, having recognised health effects, may be present at concentrations above the exemption limits (0.1% for toxic materials and 1% for all others).  These are:-

CAS No.     HAZARDOUS INGREDIENTS             %         SYMBOLS   HEALTH R PHRASES

67-63-0  	ISOPROPYL ALCOHOL  	10-25  	FXi  	R36,67

14807-96-6  	TALC, MAGNESIUM SILICATE  	<2.5

10101-41-1  	CALCIUM SULPHATE, DIHYDRATE  	50-100

Note: The text for R phrase codes shown above (if any) is given in section 16.

3. HAZARDS IDENTIFICATION

This product has been assessed under the Dangerous Preparations Directive (88/379/EEC) and is classified as follows:

: NOT CLASSIFIED

Information on Occupational Exposure Limits is given in Section 8.

4. FIRST-AID MEASURES

In all cases of doubt, or where symptoms persist, seek medical attention.

INHALATION:

Remove to fresh air, keep patient warm and at rest.  If breathing is irregular or stopped administer artificial respiration.  Give nothing by mouth.  If unconscious place in the recovery position.  Seek medical advice.

EYE CONTACT:

Irrigate copiously with clean, fresh water for at least 10 minutes, holding lids apart. Seek medical advice.

SKIN CONTACT:

Remove contaminated clothing, wash skin thoroughly with soap and water, or use a proprietary skin cleanser.  Do not use solvents or thinners.  Seek medical advice if symptoms persist.

INGESTION:

If accidentally swallowed, DO NOT INDUCE VOMITING.  Keep at rest and obtain medical attention.

5. FIRE-FIGHTING MEASURES

Extinguishing media:

Recommended - alcohol resistant foam, CO2, powders.

Not to be used - waterjet.

Recommendations    :  Fire will produce dense black smoke.  Exposure to decomposition products may cause a health hazard.  Firefighters should wear self-contained breathing apparatus. Closed containers exposed to fire should be cooled with water.  Do not allow run-off from fire-fighting to enter drains or water-courses.

6. ACCIDENTAL RELEASE MEASURES

Exclude sources of ignition and ventilate the area. Avoid breathing vapours.  Refer to protective measures listed in section 8.  Contain and collect spillage with non-combustible absorbent materials, eg sand, earth, vermiculite or diatomaceous earth, and place in container for disposal according to local regulations (see section 13).  Do not allow to enter drains or watercourses.  Clean preferably with a detergent; avoid use of solvents.  If the product contaminates lakes, rivers or sewages, inform appropriate authorities in accordance with local regulations.

7. HANDLING AND STORAGE

HANDLING CONDITIONS: Avoid contact with the skin and eyes.  Smoking, eating and drinking should be prohibited in the work area.

STORAGE CONDITIONS: Store in a cool, dry, well ventilated place and keep container tightly closed.

Water-borne products must be protected from frost and extremes of temperature.

8. EXPOSURE CONTROLS/PERSONAL PROTECTION

HAZARDOUS COMPONANTS                NDS     NDSCh    NDSP    NOTES

mg/m3    mg/m3    mg/m3

--------------------------        -------  -------  -------  ---------

ISOPROPYL ALCOHOL  	900  	1200

NDS   - Long Term Occupational Exposure Limit

NDSCh - Short Term Occupationsl Exposure Limit

NDSP  - Ceiling Occupational Exposure Limit

SUP   - Supplier Values

Further guidance on Occupational Exposure Limits and assessment of occupational exposure to harmful substances is given in Dziennik Ustaw Nr79.

RESPIRATORY PROTECTION: Avoid the inhalation of vapour, particulates and spray mist.  Where reasonably practicable, this should be achieved by the use of local exhaust ventilation and good general ventilation.  If this is not sufficient to maintain concentrations of particulates and solvent vapour below the occupational exposure limit, respiratory protection must be worn.

The selection of respiratory equipment should be in accordance with local regulations.

HAND PROTECTION: Wear suitable gloves for protection against materials in section 2.

EYE PROTECTION : Wear suitable eye protection.

SKIN PROTECTION: Wear appropriate protective clothing.  Barrier creams may help to protect exposed areas of skin but are not a substitute for full physical protection.

Dry sanding, flame cutting and/or welding of the dry paint film may give rise  to dust and/or hazardous fumes.  Wet sanding should be used wherever possible. If exposure cannot be avoided by the provision of local exhaust ventilation, suitable respiratory protective equipment should be used.

9. PHYSICAL AND CHEMICAL PROPERTIES

Physical State  : PASTE

Specific gravity: 1.450           Flash point: > 61 °C

Water miscible  : YES                       pH: 7.0-8.0

Colour: white

10. STABILITY AND REACTIVITY

CONDITIONS TO AVOID:  Extremes of temperature.

MATERIALS TO AVOID:  Keep away from oxidising agents, strongly alkaline and strongly acidic materials in order to avoid exothermic reactions.

HAZARDOUS DECOMPOSITION PRODUCTS:  When exposed to high temperatures may produce hazardous decomposition products such as carbon monoxide and dioxide, smoke and oxides of nitrogen.

11. TOXICOLOGICAL INFORMATION

There is no experimental data available on the product itself.  However, it has been assessed according to the Preparations Directive (88/379/EEC) and classified for toxicological hazards.  See Section 15 for these details, including associated risk and safety phrases.

Over-exposures of vapour are irritating to eyes and respiratory system. Excessive concentrations may produce effects on the central nervous system including drowsiness.  In extreme cases loss of consciousness may result. Long term exposure to vapour concentrations in excess of quoted OELs may result in  adverse health effects. Splashes entering the eye will cause discomfort and possible damage. Prolonged contact with the skin may have a defatting effect  which may lead to skin irritation and in some cases dermatitis.

12. ECOLOGICAL INFORMATION

There is no specific data available on the product itself.  The product should not be allowed to enter drains or watercourses or be deposited where it can affect ground or surface waters.  The Air Pollution Control requirements of regulations made under the Environmental Protection Act may apply to the use of this product.

Products classified as Marine Pollutants are indicated as such under Transport(section 14).

Substances which are classified as Dangerous for the Environment are indicated below:

13. DISPOSAL CONSIDERATIONS

Waste material and containers must be treated as a fire hazard and disposed of in accordance with the general requirements of the Control of Pollution Act 1974.

The 1990 Environmental Protection Act introduces new controls on the disposal or depositing of waste.  Due regard should be given to the "duty of care" required under the regulations and the guidance given under this act.

14. TRANSPORT INFORMATION

UN number    : 0000              Packing group: EXEMPT

ADR/RID Class: EXEMPT            Item: EXEMPT             ICI Tremcard: EXEMPT

IMDG Class   : EXEMPT            MFAG: EXEMPT             EMS: EXEMPT

Hazard label(s):

15. REGULATORY INFORMATION

NAMED SUBSTANCES: Nil

:                                  :

HAZARD CLASSIFICATION

: NOT CLASSIFIED

R67		Vapours may cause drowsiness and dizziness

S2  	Keep out of reach of children.

S51  	Use only in well ventilated areas.

S23  	Do not breathe vapour

16. OTHER INFORMATION

Text for R Phrases shown in section 2 describing each ingredient:

R36  	Irritating to eyes.

R67  	Vapours may cause drowsiness and dizziness.

The information given in this data sheet is required pursuant to EC Directive 88/379/EEC.

The information on this sheet is not a specification:  it does not guarantee specific properties.  The information is intended to provide general guidance as to health and safety based upon our knowledge of the handling, storage and use of the product.  It is not applicable to unusual or non-standard uses of the product nor where instructions and recommendations are not followed.

ICI Paints in the UK is part of Imperial Chemical Industries plc and a member of the ICI Paints World Group.  Registered in England No. 218019.

Registered office: Imperial Chemical House, Millbank, London.  SW1P 3JF.